VLAN Implementation in a Small Business Startup

In Partial Fulfilment of the Requirements in IT11/L

Submitted by:

Balase, Rodrigo F. Jr.

Elederos, Jasmine T.

Submitted to:

Lloyd Ryan Largo

Abstract

In the modern tech industry, a robust and secure network is the backbone of any IT startup. For this growing company, a reliable infrastructure is essential to support high-intensity tasks like software builds while ensuring that sensitive payroll and marketing data remain protected. Effective networking allows different departments to operate at peak performance without interfering with one another, turning basic connectivity into a strategic asset for business growth.

The company currently faces significant performance and security bottlenecks due to an unsegmented, "flat" network architecture. The Development team’s heavy bandwidth consumption for builds is causing latency for the Marketing department, hindering their ability to conduct essential video meetings. More critically, the lack of internal boundaries means that sensitive HR and Accounting data is visible to all users on the LAN. This is compounded by a default wireless configuration that leaves the office vulnerable to unauthorized access and a lack of structured routing that makes cross-departmental collaboration inefficient.

To address these challenges, the proposed solution involves segmenting the network into four distinct VLANs (Development, HR, Accounting, and Marketing) to isolate traffic and improve security. Each VLAN will be assigned a specific IP subnet to ensure organized addressing. To allow controlled communication between departments, such as HR and Accounting, we will implement inter-VLAN routing using a "router-on-a-stick" configuration with sub-interfaces. Finally, the wireless access point will be secured using WPA3 encryption and a hidden SSID or MAC filtering to prevent unauthorized connections.

The motivation behind this design is to create a professional-grade environment that balances high-speed performance with "Zero Trust" security principles. By isolating the Development team's heavy traffic, we ensure Marketing maintains the bandwidth needed for client-facing operations. Meanwhile, the VLAN segmentation provides a crucial layer of privacy for HR and Accounting, ensuring compliance and data integrity. This scalable design not only solves current congestion but also prepares the startup for future expansion without needing a complete network overhaul.

Table of Contents

Page

Project Title										   1

Abstract										   2

Table of Contents									   3

Problem Statement									   4

Problem Statement 1								   4

Problem Statement 2								   4

Problem Statement 3								   4

Goal and Objectives									   5

Specific Objective 1								   5

Specific Objective 2								   5

Specific Objective 3								   5

Network Design/Topology								   6

Configuration & Testing								   7

VLAN Configuration							      	   7

Router Configuration								   7

IP Addressing Scheme							   7

Connectivity Test Results							   7

References										   8

# Problem Statement

Problem Statement 1: Lack of VLAN Segmentation

The most immediate issue is the existence of a single, unsegmented broadcast domain. In this "flat" network, all divisions—Development, HR, Accounting, and Marketing—share the same physical and logical space. This lack of isolation creates a performance bottleneck, as high-bandwidth activities from the Development team (such as large software builds and testing) consume the available capacity, leading to network congestion for Marketing’s video meetings and file sharing. Furthermore, this creates a major security vulnerability; sensitive payroll information in accounting and employee records in HR are visible to the entire company, increasing the risk of unauthorized internal data access.

Problem Statement 2: Absence of Inter-VLAN Routing

The current network lacks a structured mechanism for controlled communication between different functional groups. While segmenting the network into VLANs is necessary for security, the organization still requires specific departments—such as HR and Accounting—to collaborate and share resources efficiently. Without configured inter-VLAN routing (such as router sub-interfaces), the network remains a collection of "islands" where data cannot move between subnets. This inefficiency forces staff to find insecure or manual workarounds to share data, defeating the purpose of a centralized network and slowing down administrative operations.

Problem Statement 3: Insecure Wireless Configuration

The third critical issue is the lack of a secure, enterprise-grade wireless configuration on the router. By leaving the wireless settings at their factory defaults, the company is exposed to external threats and unauthorized access. Default credentials and open or weak encryption protocols make it easy for individuals outside the organization to intercept data or piggyback on the company’s internet connection. This lack of security is particularly dangerous for an IT startup, as it provides a gateway for attackers to bypass physical security and access the internal resources used by Developers and the sensitive data handled by the HR and Accounting departments.

Goal and Objectives

Specific Objective 1: Implementation of VLAN Segmentation

To address the performance and security risks of an unsegmented network, the first objective is to configure and assign distinct Virtual Local Area Networks (VLANs) for the Development, HR, Accounting, and Marketing divisions. By placing each department into its own broadcast domain, we will eliminate broadcast traffic interference, ensuring that the Development team’s high-bandwidth builds do not impact Marketing’s operations. This segmentation also ensures that sensitive HR and Accounting data is logically isolated from the rest of the company.

Specific Objective 2: Configuration of Inter-VLAN Routing

To address the lack of communication between isolated departments, the second objective is to implement inter-VLAN routing using router sub-interfaces (Router-on-a-Stick configuration). This allows for regulated and secure traffic flow between specific VLANs, such as enabling Accounting and HR to share files while maintaining the overall security boundary. By configuring a single physical link to handle multiple logical subnets, the design provides an efficient path for collaboration without compromising the isolation established in the first objective.

Specific Objective 3: Deployment of Secure Wireless Access

To address the vulnerability of default wireless settings, the third objective is to configure a secure WPA2/WPA3-protected wireless environment on the router. This involves changing the default SSID, implementing strong pre-shared keys (PSK), and potentially disabling SSID broadcasting or enabling MAC address filtering. This objective ensures that only authorized laptops and mobile devices can connect to the startup’s network, protecting intellectual property and sensitive employee resources from unauthorized external access.

# Network Design/Topology

Insert your network diagram here (cisco packet tracer - screenshot).

In your explanation, include:
- List of devices used (Routers with Wi-Fi, Switches, PCs, etc.)
- VLAN assignments per department/area
- Justification for the chosen topology (Bus, Star, Ring, Mesh, etc.)
- IP Addressing plan overview

# Configuration & Testing

Provide details of your configuration and test results in the following structured format:

Example:

## 1. VLAN Configuration

Insert the configuration commands and screenshots for creating VLANs on the switch. Example:

Switch> enable
Switch# configure terminal
Switch(config)# vlan 10
Switch(config-vlan)# name Faculty

## 2. Router Configuration (Inter-VLAN + Wireless)

Insert the configuration commands for enabling inter-VLAN routing and setting up wireless. Example:

Router> enable
Router# configure terminal
Router(config)# interface g0/0.10
Router(config-subif)# encapsulation dot1Q 10
Router(config-subif)# ip address 192.168.10.1 255.255.255.0

## 3. IP Addressing Scheme

Fill in the table below to show your complete IP addressing plan:

| Department / VLAN | Network Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| Faculty (VLAN 10) | 192.168.10.0/24 | 255.255.255.0 | 192.168.10.1 |
| Laboratory (VLAN 20) | 192.168.20.0/24 | 255.255.255.0 | 192.168.20.1 |
| Library (VLAN 30) | 192.168.30.0/24 | 255.255.255.0 | 192.168.30.1 |

## 4. Connectivity Test Results

Provide proof of successful connectivity:
- Ping tests between VLANs (screenshots)
- Verification that access restrictions are working as intended

# References

Book (APA 7th)
Cisco Networking Academy. (2021). Introduction to networks companion guide. Cisco Press.

Website (APA 7th)
Cisco. (2024, March 15). How to configure a wireless router in Packet Tracer. Cisco Networking Academy. https://www.netacad.com/resources/labs